# V2X Robot Demonstration Platform: Technical Architecture & Execution Manual

## 1. Executive Summary & Project Objective

This document details the complete design, hardware allocation, software stack, and execution roadmap for a small-scale Vehicle-to-Everything (V2X) robotics demonstration platform [cite: 1]. The goal of this project is to visually and technically demonstrate cooperative autonomy, real-time vehicle-to-vehicle communication, and emergency vehicle priority handling in a controlled indoor environment [cite: 1].

The system features two autonomous tracked or wheeled robots moving along a shared loop [cite: 1]:

- The Car (Host Vehicle / HV): Operates as a standard autonomous vehicle traveling at a baseline cruising speed [cite: 1].

- The Ambulance (Emergency Vehicle / EV): Operates as a priority vehicle traveling at a higher speed [cite: 1].

When the Ambulance approaches from behind, it broadcasts an emergency V2X message [cite: 1]. The Car receives this packet, identifies a safe yielding zone via localized physical checkpoints, and dynamically executes a lateral lane-shift maneuver to let the Ambulance pass before safely returning to the primary travel path [cite: 1].

## 2. Physical Infrastructure & Track Design

The entire track is software-defined and built atop an inexpensive, modular, and portable flooring setup [cite: 1]. Rather than constructing physical boundaries or rigid lanes, the layout uses logical markings that the robots interpret using computer vision [cite: 1].

### 2.1 Grid Specifications

| Parameter | Specification |
| --- | --- |
| Platform Configuration | 4 × 4 Matrix (16 interlocking foam mats total) [cite: 1] |
| Individual Mat Size | 60 cm × 60 cm [cite: 1] |
| Total Platform Footprint | 240 cm × 240 cm (2.4m × 2.4m) [cite: 1] |
| Track Geometry | Rounded Square Loop (maximizing full surface area) [cite: 1] |
| Corner Turn Radius | 45–50 cm (optimized to eliminate wheel scrubbing and slip) [cite: 1] |

### 2.2 Surface Markings & Visual Coding

- Solid Black Lines: Mark the inner and outer boundaries of the drivable road surface [cite: 1]. The robots utilize these lines to establish their maximum lateral boundaries [cite: 1].

- Dashed White Line (Centerline): Placed in the center of the road [cite: 1]. This represents the default 0-cm offset travel path for both vehicles under normal operating conditions [cite: 1].

- Dashed Yellow Line (Side Yield Zone): Applied only along the straightaway sections of the loop, offset to the right of the centerline [cite: 1]. This outlines the virtual lane target for the Car during an active yielding maneuver [cite: 1].

### 2.3 Localization Matrix (QR Codes)

A series of 16 unique QR codes or AprilTags are physically mounted vertically along the perimeter of the outer black boundary, spaced roughly 20 cm apart [cite: 1]. These markers function as absolute coordinate checkpoints [cite: 1]. Rather than relying heavily on error-prone wheel odometry over time, reading a specific QR code tells the robot exactly which segment of the track it is currently traversing (e.g., Corner 2, Straightaway 1, or Yield-Capable Zone) [cite: 1].

## 3. Hardware Architecture

Both robots share an identical Master-Slave dual-processor architecture [cite: 1]. This split-processing framework isolates high-overhead visual tasks from time-critical motor loops, guaranteeing system stability and preventing real-time control lag [cite: 1].

### 3.1 Component Breakdown

- The Brain: Raspberry Pi (Model 4B or 5 recommended) [cite: 1]

  - Responsible for processing the video stream via OpenCV [cite: 1].

  - Detects the white centerline, boundaries, and reads QR checkpoints [cite: 1].

  - Manages the V2X wireless networking stack via local Wi-Fi [cite: 1].

  - Computes path-tracking steering commands using the Pure Pursuit algorithm [cite: 1].

- The Vision System: Raspberry Pi CSI Camera Module [cite: 1]

  - Rigidly mounted to the front chassis, angled downward toward the floor [cite: 1].

  - Connected via a direct CSI ribbon cable directly into the Pi's internal Image Signal Processor (ISP), ensuring sub-millisecond video pipeline latency and high frame rates (60+ FPS at 640×480 resolution) [cite: 1].

- The Muscle: STM32 Microcontroller (e.g., STM32F4 or STM32F1 series) [cite: 1]

  - Handles low-level hardware interactions [cite: 1].

  - Reads raw quadrature encoder ticks from the DC motors to compute real-time velocity [cite: 1].

  - Executes a high-frequency proportional-integral-derivative (PID) loop to match target motor speeds [cite: 1].

  - Outputs precise pulse-width modulation (PWM) signals to the H-Bridge motor drivers [cite: 1].

- Communication & Power: [cite: 1]

  - Inter-Processor Comm: High-speed hardware UART (Serial) connection linking the Raspberry Pi to the STM32 [cite: 1].

  - V2X Link: Standard onboard Wi-Fi utilized to establish peer-to-peer UDP or local socket connections [cite: 1].

## 4. Software Architecture & Control Logic

The software stack avoids the structural complexity of ROS2, opting instead for a highly efficient, single-threaded Python pipeline on the Raspberry Pi and a lightweight C script running on the STM32 [cite: 1].

### 4.1 Computer Vision Pipeline (OpenCV)

- Region of Interest (ROI) Cropping: The raw frame from the angled CSI camera is cropped to eliminate horizon noise and isolate the road surface immediately in front of the vehicle [cite: 1].

- Perspective Transform (Bird's-Eye View): A homography matrix maps the angled perspective into a flat, linear coordinate frame, eliminating perspective distortion where lines converge in the distance [cite: 1].

- Thresholding: The frame is converted to grayscale and binarized to separate the bright white centerline and yellow markings from the gray foam background [cite: 1].

- Error Extraction: The algorithm calculates the horizontal pixel offset ($e_y$) between the camera's center axis and the detected white centerline [cite: 1].

import cv2
import numpy as np

def get_birds_eye_view(frame):
    h, w = frame.shape[:2]
    src_points = np.float32([
        [int(w*0.25), int(h*0.6)], 
        [int(w*0.75), int(h*0.6)], 
        [w, h],                    
        [0, h]                     
    ])
    dst_points = np.float32([
        [0, 0],
        [w, 0],
        [w, h],
        [0, h]
    ])
    M = cv2.getPerspectiveTransform(src_points, dst_points)
    warped = cv2.warpPerspective(frame, M, (w, h))
    return warped

def calculate_line_error(warped_frame):
    gray = cv2.cvtColor(warped_frame, cv2.COLOR_BGR2GRAY)
    _, thresh = cv2.threshold(gray, 200, 255, cv2.THRESH_BINARY)
    scan_row = int(thresh.shape[0] * 0.7)
    row_pixels = thresh[scan_row, :]
    white_indices = np.where(row_pixels == 255)[0]
    
    if len(white_indices) > 0:
        line_center = int(np.mean(white_indices))
        camera_center = thresh.shape[1] // 2
        error = line_center - camera_center
        return error
    return None

### 4.2 Path Tracking: Pure Pursuit

The steering control is dictated by a modified Pure Pursuit algorithm [cite: 1]. The Raspberry Pi designates a virtual look-ahead distance ($L_t$) along the detected centerline path [cite: 1].

- Under normal states, the steering angle is calculated to drive the robot directly toward a target point centered on the white line ($\Delta Y = 0\text{ cm}$) [cite: 1].

- When a yielding maneuver is active, a software offset is introduced, shifting the target point to the right ($\Delta Y = +15\text{ cm}$) [cite: 1]. The Pure Pursuit controller naturally calculates a smooth steering arc to merge the robot onto the yellow dashed line without modifying the underlying path tracking script [cite: 1].

def pure_pursuit_control(cross_track_error, target_offset, look_ahead_dist=120):
    if cross_track_error is None:
        return 0.0  
        
    effective_error = cross_track_error - target_offset
    steering_angle = (2.0 * effective_error) / look_ahead_dist
    steering_angle = np.clip(steering_angle, -30, 30)
    return int(steering_angle)

### 4.3 Low-Latency V2X Communication Layer (UDP Sockets)

To achieve low latency, the system utilizes non-blocking UDP sockets over local Wi-Fi.

#### Ambulance Broadcast Script:

import socket
import json

V2X_IP = "255.255.255.255" 
V2X_PORT = 5005
sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
sock.setsockopt(socket.SOL_SOCKET, socket.SO_BROADCAST, 1)

def broadcast_emergency_status(current_qr_id, speed):
    payload = {
        "vehicle_type": "ambulance",
        "status": "EMERGENCY_APPROACHING",
        "last_qr": current_qr_id,
        "speed_cms": speed
    }
    message = json.dumps(payload).encode('utf-8')
    sock.sendto(message, (V2X_IP, V2X_PORT))

#### Car Receiver Script:

import socket
import json

V2X_PORT = 5005
sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
sock.bind(("0.0.0.0", V2X_PORT))
sock.setblocking(0)

def check_v2x_signals():
    try:
        data, addr = sock.recvfrom(1024)
        payload = json.loads(data.decode('utf-8'))
        if payload.get("status") == "EMERGENCY_APPROACHING":
            return True, payload["last_qr"]
    except BlockingIOError:
        pass
    return False, None

### 4.4 System State Machines & Coordination Loop

The cooperative behavior relies on two asynchronous state machines interacting over the Wi-Fi socket connection [cite: 1].

#### Ambulance State Machine:

[State: NORMAL_CRUISE] [cite: 1]
       │
       ▼ (Reads QR Checkpoint near the Car's rear sector) [cite: 1]
[State: V2X_BROADCAST] ──► (Transmits UDP packet) [cite: 1]
       │
       ▼ (Detects Car has cleared the lane via QR matrix) [cite: 1]
[State: OVERTAKE] ──► (Increases velocity, passes Car on centerline) [cite: 1]
       │
       ▼ (Clears the straightaway sector) [cite: 1]
[State: RESUME_NORMAL] [cite: 1]

#### Car State Machine:

[State: DEFAULT_FOLLOW] (Target Offset: ΔY = 0) [cite: 1]
       │
       ▼ (Receives V2X Broadcast Packet AND reads QR Tag) [cite: 1]
[State: EXECUTE_YIELD] (Target Offset shifts to ΔY = +15cm; slows down velocity) [cite: 1]
       │
       ▼ (Receives V2X Clear Packet OR notes Ambulance is safely ahead) [cite: 1]
[State: RECOVER_PATH] (Target Offset glides smoothly back to ΔY = 0) [cite: 1]

#### Main Execution Loop (Car):

current_state = "DEFAULT_FOLLOW"
target_offset = 0
base_cruise_speed = 20 

while True:
    frame = camera.read_mipi_frame()
    warped = get_birds_eye_view(frame)
    cross_track_error = calculate_line_error(warped)
    current_qr = scan_qr_checkpoints(warped)
    ambulance_detected, ambulance_qr_id = check_v2x_signals()
    
    if current_state == "DEFAULT_FOLLOW":
        target_offset = 0
        speed_command = base_cruise_speed
        if ambulance_detected and current_qr == "YIELD_ZONE_START":
            current_state = "EXECUTE_YIELD"
            
    elif current_state == "EXECUTE_YIELD":
        target_offset = 45  
        speed_command = base_cruise_speed * 0.5  
        if ambulance_qr_id is not None and current_qr is not None:
            if int(ambulance_qr_id) > int(current_qr): 
                current_state = "RECOVER_PATH"
                
    elif current_state == "RECOVER_PATH":
        target_offset = 0
        speed_command = base_cruise_speed
        if abs(cross_track_error) < 5: 
            current_state = "DEFAULT_FOLLOW"

    steer_command = pure_pursuit_control(cross_track_error, target_offset)
    uart_serial.write(f"DRIVE,{speed_command},{steer_command}\n")

## 5. Low-Level Control: STM32 C Firmware

The STM32 parses the UART command pipeline (DRIVE,[Velocity],[Steering_Angle]\n) and handles high-frequency (100Hz) dual-channel wheel velocity PID loops.

#include "stm32f4xx_hal.h"
#include <stdio.h>
#include <string.h>

#define KP 1.5f
#define KI 0.2f
#define KD 0.05f

float target_velocity_left = 0.0f;
float target_velocity_right = 0.0f;
float current_velocity_left = 0.0f;
float current_velocity_right = 0.0f;

void compute_differential_speeds(float V, float delta) {
    float track_width = 20.0f; 
    target_velocity_left  = V - (delta * (track_width / 2.0f) * 0.1f);
    target_velocity_right = V + (delta * (track_width / 2.0f) * 0.1f);
}

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim) {
    if (htim->Instance == TIM6) { 
        int16_t encoder_ticks_left = (int16_t)__HAL_TIM_GET_COUNTER(&htim2);
        int16_t encoder_ticks_right = (int16_t)__HAL_TIM_GET_COUNTER(&htim3);
        
        __HAL_TIM_SET_COUNTER(&htim2, 0);
        __HAL_TIM_SET_COUNTER(&htim3, 0);
        
        current_velocity_left = (float)encoder_ticks_left * 0.05f; 
        current_velocity_right = (float)encoder_ticks_right * 0.05f;
        
        static float error_integral_l = 0, last_error_l = 0;
        float error_l = target_velocity_left - current_velocity_left;
        error_integral_l += error_l * 0.01f;
        float derivative_l = (error_l - last_error_l) / 0.01f;
        float output_pwm_l = (KP * error_l) + (KI * error_integral_l) + (KD * derivative_l);
        last_error_l = error_l;
        
        __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, (int)output_pwm_l);
    }
}

void parse_uart_command(char* rx_buffer) {
    if (strncmp(rx_buffer, "DRIVE", 5) == 0) {
        float velocity = 0.0f;
        float steering_angle = 0.0f;
        sscanf(rx_buffer, "DRIVE,%f,%f", &velocity, &steering_angle);
        compute_differential_speeds(velocity, steering_angle);
    }
}

## 6. Step-by-Step Execution Plan

### Phase 1: Mechanical and Electrical Assembly

- Assemble the two chassis platforms [cite: 1]. Rigidly mount the CSI cameras at a fixed downward angle (approx. 30–45 degrees from the vertical axis) to prevent frame vibration [cite: 1].

- Wire the DC motors to the H-bridge drivers, route the quadrature encoder pins to the STM32 hardware timer inputs, and connect the STM32 to the Raspberry Pi via standard UART pins (TX/RX cross-connected) [cite: 1].

### Phase 2: Low-Level Motor Tuning (STM32)

- Flash the STM32 firmware to read incoming serial strings structured as: DRIVE,linear_velocity,steering_angle\n [cite: 1].

- Tune the internal PID velocity controller loop using step-response metrics to ensure the wheels lock onto target speeds cleanly with minimal overshoot, regardless of battery voltage fluctuations [cite: 1].

### Phase 3: Vision and Control Tuning (Raspberry Pi)

- Set up the 4×4 foam mat layout and tape down a test straightaway and corner segment using the chosen colored tapes [cite: 1].

- Calibrate the OpenCV script's perspective transform by placing a physical grid on the track to map pixel coordinates to real-world centimeters [cite: 1].

- Implement the Pure Pursuit loop [cite: 1]. Test single-robot line tracking at low speeds, gradually raising velocity while tuning the look-ahead distance ($L_t$) to eliminate lateral oscillations on the corners [cite: 1].

### Phase 4: Communication and V2X Integration

- Configure both Raspberry Pis to automatically connect to a shared local Wi-Fi router upon bootup, assigning fixed static IP addresses to each vehicle [cite: 1].

- Write a lightweight Python socket script using the socket library to transmit and listen for non-blocking UDP packets over a designated port [cite: 1].

- Verify that the Car instantly registers the broadcast packet from the Ambulance while simultaneously running its lane-following computer vision loop [cite: 1].

### Phase 5: Full-Scale Demonstration Tracking

- Deploy the entire 4×4 mat track loop with all 16 QR checkpoints fastened securely along the boundaries [cite: 1].

- Place both robots on the centerline, initiating the default cruise speeds [cite: 1].

- Activate the Ambulance's fast-tracking routine, prompting the automated V2X message transmission and validating the Car's real-time lateral shift, slowdown, priority pass capture, and clean re-entry tracking [cite: 1].